Type H12

| Model | H12 -200 | H12 -300 | H12 -400 | H12 -500 | H12 -600 | H12 -800 | H12 -1000 | H12 -1500 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rated power | 200w | 300w | 400w | 500w | 600w | 800w | 1kw | 1.5kw |
| Rated voltage | 12V/24V/48V |  |  |  | 24v/48v/96v |  |  |  |
| Start the wind speed | 2m/s |  |  |  |  |  |  |  |
| Rated wind speed | 12m/s |  |  |  |  |  |  |  |
| Safe wind speed | ≤50m/s |  |  |  |  |  |  |  |
| Blade diameter/height（ m） | 0.75/0.8 | 0.75/1 | 0.75/1.2 | 0.75/1.4 | 0.95/1.4 | 0.95/1.6 | 1.28/1.8 | 1.28/2 |
| Number of blades | 5 |  |  |  |  |  |  |  |
| Blade material | Aluminum alloy |  |  |  |  |  |  |  |
| Fuselage material | Aluminum alloy |  |  |  |  |  |  |  |
| Magnetic steel material | NdFeB |  |  |  |  |  |  |  |
| Generator type | Three-phase AC permanent magnet synchronous generator |  |  |  |  |  |  |  |
| Control mode | Electromagnetic brakes |  |  |  |  |  |  |  |
| Wind direction adjustment | 360° automatic windward wind |  |  |  |  |  |  |  |
| Service life/Degree of protection | ≥ 20 yearsIP54 |  |  |  |  |  |  |  |
| Net weight/gross weight（kg） | 15/17 | 16/18 | 17/19 | 18/20 | 21/23 | 23/25 | 75/90 | 85/100 |
| Packing size（ cm） | 35*34*22+ 85*26*26 | 35*34*22+ 110*26*26 | 35*34*22+ 130*26*26 | 19*19*21+ 147*26*26 | 36*35*22+ 147*26*26 | 36*35*22+ 162*26*26 | 31*31*27+ 185*25*35 | 31*31*27+ 205*25*35 |

Type H12

| Model | H12 -2K | H12 -3K | H12 -5K | H12 -10K |  | H12 -20K |  | H12 -30K | H12 -50K | H12 -100K |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rated power | 2000w | 3000w | 5000w | 10Kw |  | 20Kw |  | 30Kw | 50kw | 100kw |
| Rated voltage | 48V/96V/120V/220V/380V |  |  | 120V/220V/240V/380V/480V |  |  |  |  | 380V/480V/600V… … |  |
| Start the wind speed | 2m/s |  |  | 2.5m/s |  |  |  |  | 3m/s |  |
| Rated wind speed | 12m/s |  |  | 11m/s |  |  |  |  | 10m/s |  |
| Safe wind speed | ≤50m/s |  |  |  |  |  |  |  |  |  |
| Blade diameter/height（ m） | 1.55/2.4 | 2/3 | 2.2/4.2 | 3.9/5 |  | 4.65/7 |  | 5.8/10 | 6.65/15 | 9/25 |
| Number of blades | 5 |  |  |  |  |  |  |  |  |  |
| Blade material | Aluminum alloy |  |  |  |  |  |  |  |  |  |
| Fuselage material | Aluminum alloy |  | cast iron |  |  |  |  |  |  |  |
| Magnetic steel material | NdFeB |  |  |  |  |  |  |  |  |  |
| Generator type | Three -phase AC external rotor generator |  |  |  |  |  |  |  |  |  |
| Control mode | Electromagnetic brakes |  |  |  |  |  |  |  |  |  |
| Wind direction adjustment | 360° automatic windward wind |  |  |  |  |  |  |  |  |  |
| Service life/Degree of protection | ≥ 20 yearsIP54 |  |  |  |  |  |  |  |  |  |
| Net weight（kg） | 150 | 280 | 650 |  | ≈ 1.3t |  | - |  |  |  |
| Packing size （ m） | 2.5*0.33*0.32+ 1.1*0.4*0.48 | 1.5*0.5*0.5+ 3.1*0.4*0.4 | 4.25*0.32*0.52 1.3*0.66*0.67+ 2.3*0.6*0.6 |  | - |  |  |  |  |  |

Type H530 plus

| Model | H530-400 | H530-500 | H530-600 | H530 -800 | H530 -1000 | H530-1500 |
| --- | --- | --- | --- | --- | --- | --- |
| Rated power | 400W | 500W | 600W | 800W | 1000W | 1500W |
| Rated voltage | 12V/24V/48V |  |  |  | 24V/48V/96V |  |
| Start the wind speed | 2.5m/s |  |  |  |  |  |
| Rated wind speed | 11m/s |  |  |  |  |  |
| Safe wind speed | ≤50m/s |  |  |  |  |  |
| Blade diameter/height（ m） | 0.95*1.3 | 0.95*1.4 | 1.16*1.6 | 1.16*1.8 | 1.35*2 | 1.67*2.2 |
| Number of blades | 5 high-speed blades+Assist the wind wheel |  |  |  |  |  |
| Blade material | Aluminum alloy |  |  |  |  |  |
| Fuselage material | Aluminum alloy |  |  |  |  |  |
| Magnetic steel material | NdFeB |  |  |  |  |  |
| Generator type | Three -phase AC external rotor generator |  |  |  |  |  |
| Control mode | Electromagnetic brakes |  |  |  |  |  |
| Wind direction adjustment | 360° automatic windward wind |  |  |  |  |  |
| Service life/Degree of protection | ≥ 20 yearsIP54 |  |  |  |  |  |
| Net weight（kg） |  |  |  |  |  |  |
| Packing size（ m） | 1*0.34*0.38+ 1.3*0.25*0.3 | 1*0.34*0.38+ 1.4*0.25*0.3 | 1.5*0.5*0.6+ 1.65*0.3*0.3 | 1.5*0.5*0.6+ 1.85*0.3*0.3 | 1.6*0.5*0.5+ 2.05*0.31*0.31 | 1.6*0.7*0.81+ 2.25*0.32*0.32 |

Type H530 plus

| Model | H530-2000 | H530-3000 | H530-5000 | H530 -10K | H530 -20K | H530-30K |
| --- | --- | --- | --- | --- | --- | --- |
| Rated power | 2000W | 3000W | 5000W | 10KW | 20KW | 30KW |
| Rated voltage | 48V/96V/120V/220V/380V |  |  | 220V/380V/480V… … |  |  |
| Start the wind speed | 2.5m/s |  |  |  |  |  |
| Rated wind speed | 11m/s |  |  |  |  |  |
| Safe wind speed | ≤50m/s |  |  |  |  |  |
| Blade diameter/height（ m） | 2.1/3.2 | 2.63*4.2 | 3.15*5 | 4.25*6.8 | 6.05*9.5 | 7.5*11.5 |
| Number of blades | 5 high-speed blades+Assist the wind wheel |  |  |  |  |  |
| Blade material | Aluminum alloy |  |  |  |  |  |
| Fuselage material | Aluminum alloy |  | cast iron |  |  |  |
| Magnetic steel material | NdFeB |  |  |  |  |  |
| Generator type | Three -phase AC external rotor generator |  |  |  |  |  |
| Control mode | Electromagnetic brakes |  |  |  |  |  |
| Wind direction adjustment | 360° automatic windward wind |  |  |  |  |  |
| Service life/Degree of protection | ≥ 20 yearsIP54 |  |  |  |  |  |
| Net weight（kg） |  |  | 850kg |  |  |  |
| Packing size（ m） |  |  | 3.25*1*1+ 5*0.7*0.8+ 1.7*0.6*0.35 |  |  |  |

Type HS530                                        Type HS310

| Model | HS-100 | HS-200 | HS-300 | HS-400 |
| --- | --- | --- | --- | --- |
| Rated power | 100W | 200W | 300W | 400W |
| Rated voltage | 12V/24V |  |  |  |
| Start the wind speed | 1.5m/s |  |  |  |
| Rated wind speed | 12m/s |  |  |  |
| Safe wind speed | ≤50m/s |  |  |  |
| Fan blade diameter/height | 0.55m/0.8m |  |  |  |
| Number of blades | 3/5 |  |  |  |
| Blade material | nylon |  |  |  |
| Fuselage material | Aluminum alloy |  |  |  |
| Magnetic steel material | NdFeB |  |  |  |
| Generator type | Three -phase AC permanent magnet levitation generator |  |  |  |
| Control mode | Electromagnetic brakes |  |  |  |
| Wind direction adjustment | 360° automatic windward wind |  |  |  |
| Service life/Degree of protection | ≥ 20 yearsIP54 |  |  |  |
| Net weight/gross weight | 9.5kg/10.5kg | 10kg/11kg | 10.5kg/11.5kg | 11kg/12kg |
| Packing size | 86*38*20cm |  |  |  |

Type HLK33

| Model | HLK33 -400 | HLK33 -500 | HLK33 -600 | HLK33 -800 | HLK33 -1000 | HLK33 -1500 |
| --- | --- | --- | --- | --- | --- | --- |
| Rated power | 400w | 500w | 600w | 800w | 1000w | 1500w |
| Rated voltage | 12V/24V/48v/96v |  |  |  |  |  |
| Start the wind speed | 2.5m/s |  |  |  |  |  |
| Rated wind speed | 12m/s |  |  |  |  |  |
| Safe wind speed | ≤50m/s |  |  |  |  |  |
| Blade diameter/height（ m） | 0.78*1.3 | 0.78*1.4 | 0.95*1.6 | 0.95*1.8 | 1.16*2 | 1.35*2.2 |
| Number of blades | 3 |  |  |  |  |  |
| Blade material | Aluminum alloy |  |  |  |  |  |
| Fuselage material | Aluminum alloy |  |  |  |  |  |
| Magnetic steel material | NdFeB |  |  |  |  |  |
| Generator type | Three -phase AC External rotor generator |  |  |  |  |  |
| Control mode | Electromagnetic brakes |  |  |  |  |  |
| Winddirection adjustment | Automatic windward |  |  |  |  |  |
| Design life | 25 years |  |  |  |  |  |
| Degree of protection | IP54 |  |  |  |  |  |

Type Q1                                               Type Q2

| Model | Q1/Q2 -100 | Q1/Q2 -200 | Q1/Q2 -300 |
| --- | --- | --- | --- |
| Rated power | 100W | 200W | 300W |
| Rated voltage | 12V/24V |  |  |
| Start the wind speed | 2m/s |  |  |
| Rated wind speed | 12m/s |  |  |
| Safe wind speed | ≤50m/s |  |  |
| Fan blade diameter/height | 1*0.9 |  |  |
| Number of blades | 3 high-speed blades+Assist the wind wheel |  |  |
| Blade material | Aluminum alloy |  |  |
| Fuselage material | Aluminum alloy |  |  |
| Magnetic steel material | NdFeB |  |  |
| Generator type | Three -phase AC permanent magnet levitation generator |  |  |
| Control mode | Electromagnetic brakes |  |  |
| Wind direction adjustment | 360° automatic windward wind |  |  |
| Service life/Degree of protection | ≥ 20 yearsIP54 |  |  |
| Net weight/gross weight | 16kg/17kg | 16.5kg/17.5kg | 17kg/18kg |
| Packing size | 910*330*330mm+900*500*100mm |  |  |

Type Q4max                                           Type Q5max

| Model | Q4-400 | Q4-500 | Q5-600 | Q5-800 | Q5-1000 |
| --- | --- | --- | --- | --- | --- |
| Rated power | 400W | 500W | 600W | 800W | 1000W |
| Rated voltage | 12V/24V/48V |  | 24V/48V/96V |  |  |
| Start the wind speed | 2m/s |  |  |  |  |
| Rated wind speed | 12m/s |  |  |  |  |
| Safe wind speed | ≤50m/s |  |  |  |  |
| Fan blade diameter/height | 1.15m/1m |  | 1.3m/1.2m |  | 1.6m/1.5m |
| Number of blades | 3 high-speed blades+Assist the wind wheel |  |  |  |  |
| Blade material | Aluminum alloy |  |  |  |  |
| Fuselage material | Aluminum alloy |  |  |  |  |
| Magnetic steel material | NdFeB |  |  |  |  |
| Generator type | Three -phase AC external rotor generator |  |  |  |  |
| Control mode | Electromagnetic brakes |  |  |  |  |
| Wind direction adjustment | 360° automatic windward wind |  |  |  |  |
| Service life/Degree of protection | ≥ 20 yearsIP54 |  |  |  |  |
| Net weight/gross weight | 26kg/28kg | 28kg/30kg | 40kg/50kg | 45kg/55kg | 85kg/100kg |
| Packing size | 1.13*0.43*0.43m+0.97*0.12*0.66m |  | 1.22*0.5*0.6m+1.2*0.6*0.1m |  | 1.6*0.7*0.7m+ 1.5*0.8*0.1 |

Type Q10max

| Model | Q10-1500 | Q10-2000 | Q10-3000 | Q10 -5000 |
| --- | --- | --- | --- | --- |
| Rated power | 1500W | 2000W | 3000W | 5000W |
| Rated voltage | 48V/96V/120V/220V/380V |  |  |  |
| Start the wind speed | 2.5m/s |  |  |  |
| Rated wind speed | 12m/s |  |  |  |
| Safe wind speed | ≤50m/s |  |  |  |
| Fan blade diameter/height | 2m/1.9m | 2.5m/2.4m | 3m/2.9m | 3.6m/3.5m |
| Number of blades | 3 high-speed blades+Assist the wind wheel |  |  |  |
| Blade material | Aluminum alloy |  |  |  |
| Fuselage material | Aluminum alloy |  |  |  |
| Magnetic steel material | NdFeB |  |  |  |
| Generator type | Three -phase AC external rotor generator |  |  |  |
| Control mode | Electromagnetic brakes |  |  |  |
| Wind direction adjustment | 360° automatic windward wind |  |  |  |
| Service life/Degree of protection | ≥ 20 yearsIP54 |  |  |  |
| Net weight/gross weight |  | 200kg/270kg |  |  |
| Packing size | 2*0.7*0.7m+ 1.5*1*0.15m | 2.4*0.85*0.85m+ 2*1.4*0.2m | 3.2*1.03*1.14m+ 2.5*1.5*0.25m | 3.8*1.2*1.3m+ 3*2*0.3m |

Type X3                                Type X3plus

| Model | X3-100 | X3-200 | X3-300 | X3 -400 |
| --- | --- | --- | --- | --- |
| Rated power | 100W | 200W | 300W | 400W |
| Rated voltage | 12V/24V |  |  |  |
| Start the wind speed | 2m/s |  |  |  |
| Rated wind speed | 12m/s |  |  |  |
| Safe wind speed | ≤50m/s |  |  |  |
| Fan blade diameter/height | 0.55m/0.75m |  |  |  |
| Number of blades | 3 |  |  |  |
| Blade material | nylon |  |  |  |
| Fuselage material | Aluminum alloy |  |  |  |
| Magnetic steel material | NdFeB |  |  |  |
| Generator type | Three -phase AC permanent magnet levitation generator |  |  |  |
| Control mode | Electromagnetic brakes |  |  |  |
| Wind direction adjustment | 360° automatic windward wind |  |  |  |
| Service life/Degree of protection | ≥ 20 yearsIP54 |  |  |  |
| Net weight/gross weight | 9kg/10kg | 9.5kg/10.5kg | 10kg/11kg | 11kg/12kg |
| Packing size | 86*38*20cm |  |  |  |

Type X5                                         Type X5 plus

| Model | X5 -400 | X5-500 | X5-600 |
| --- | --- | --- | --- |
| Rated power | 400W | 500W | 600W |
| Rated voltage | 12V/24V/48V |  |  |
| Start the wind speed | 2m/s |  |  |
| Rated wind speed | 12m/s |  |  |
| Safe wind speed | ≤50m/s |  |  |
| Fan blade diameter/height | 0.65m/0.9m |  |  |
| Number of blades | 3 |  |  |
| Blade material | nylon |  |  |
| Fuselage material | Aluminum alloy |  |  |
| Magnetic steel material | NdFeB |  |  |
| Generator type | Three -phase AC permanent magnet levitation generator |  |  |
| Control mode | Electromagnetic brakes |  |  |
| Wind direction adjustment | 360° automatic windward wind |  |  |
| Service lifeDegree of protection | ≥ 20 yearsIP54 |  |  |
| Net weight/gross weight | 20kg/21kg | 21kg/22kg | 22kg/23kg |
| Packing size | 106*27*49cm |  |  |

Type X9

| Model | X9 -800 | X9-1000 | X9-1500 |
| --- | --- | --- | --- |
| Rated power | 800W | 1000W | 1500W |
| Rated voltage | 24V/48V/96V |  |  |
| Start the wind speed | 2m/s |  |  |
| Rated wind speed | 12m/s |  |  |
| Safe wind speed | ≤50m/s |  |  |
| Fan blade diameter/height | 0.8m/1.5m |  | 1.1m/1.65m |
| Number of blades | 3 |  |  |
| Blade material | Reinforced FRP |  | Aluminum alloy |
| Fuselage material | Aluminum alloy |  |  |
| Magnetic steel material | NdFeB |  |  |
| Generator type | Three -phase AC permanent magnet levitation generator |  |  |
| Control mode | Electromagnetic brakes |  |  |
| Wind direction adjustment | 360° automatic windward wind |  |  |
| Service life/Degree of protection | ≥ 20 yearsIP54 |  |  |
| Net weight/gross weight |  |  |  |
| Packing size | 100*37*41cm+220*30*50cm |  | 100*52*57cm+220*30*50cm |

Small spiral type - permanent magnet

| Model | SSV-100 | SSV-200 |
| --- | --- | --- |
| Rated power | 100W | 200W |
| Rated voltage | 12V/24V |  |
| Start the wind speed | 1.5m/s |  |
| Rated wind speed | 12m/s |  |
| Safe wind speed | ≤50m/s |  |
| Fan blade diameter/height | 0.4m/0.7m |  |
| Number of blades | 2 |  |
| Blade material | Aluminum alloy |  |
| Fuselage material | Aluminum alloy |  |
| Magnetic steel material | NdFeB |  |
| Generator type | Three -phase AC permanent magnet generator |  |
| 控制方式Control mode | Electromagnetic brakes |  |
| Wind direction adjustment | 360° automatic windward wind |  |
| Service life/Degree of protection | ≥ 20 yearsIP54 |  |
| Net weight/gross weight | 8kg/9kg | 9kg/10kg |
| Packing size | 73*22*20cm |  |

Small spiral type - permanent magnet levitation    generator

| Model | SV-100 | SV-200 | SV-300 |
| --- | --- | --- | --- |
| Rated power | 100W | 200W | 300W |
| Rated voltage | 12V/24V |  |  |
| Start the wind speed | 1.5m/s |  |  |
| Rated wind speed | 12m/s |  |  |
| Safe wind speed | ≤50m/s |  |  |
| Fan blade diameter/height | 0.5m/1m |  | 0.5m/1.15m |
| Number of blades | 10/2 |  | 12 |
| Blade material | Aluminum alloy |  |  |
| Fuselage material | Aluminum alloy |  |  |
| Magnetic steel material | NdFeB |  |  |
| Generator type | Three -phase AC permanent magnet levitation generator |  |  |
| Control mode | Electromagnetic brakes |  |  |
| Wind direction adjustment | 360° automatic windward wind |  |  |
| Service life/Degree of protection | ≥ 20 yearsIP54 |  |  |
| Net weight/gross weight | 13kg/14kg | 14kg/15kg | 16kg/17kg |
| Packing size | 1000*260*260mm |  | 1200*260*260mm |

Lager spiral type - ermanent magnet levitation   generator

| Model | SV-200 | SV-300 | SV-400 |
| --- | --- | --- | --- |
| Rated power | 200W | 300W | 400W |
| Rated voltage | 12V/24V |  |  |
| Start the wind speed | 1.5m/s |  |  |
| Rated wind speed | 12m/s |  |  |
| Safe wind speed | ≤50m/s |  |  |
| Fan blade diameter/height | 0.65m/1.25m |  | 0.65m/1.45m |
| Number of blades | 10 |  | 12 |
| Blade material | Aluminum alloy |  |  |
| Fuselage material | Aluminum alloy |  |  |
| Magnetic steel material | NdFeB |  |  |
| Generator type | Three -phase AC permanent magnet levitation generator |  |  |
| Control mode | Electromagnetic brakes |  |  |
| Wind direction adjustment | 360° automatic windward wind |  |  |
| Service life/Degree of protection | ≥ 20 yearsIP54 |  |  |
| Net weight/gross weight | 21kg/22kg | 22kg/23kg | 26kg/27kg |
| Packing size | 1470*270*270mm |  | 1620*270*270mm |

Type V/Lager spiral type - ermanent magnet levitation   generator

| Model | V-800 | V-1000 | V-1500 |
| --- | --- | --- | --- |
| Rated power | 800W | 1000W | 1500W |
| Rated voltage | 24V/48V/96V |  |  |
| Start the wind speed | 2m/s |  |  |
| Rated wind speed | 12m/s |  |  |
| Safe wind speed | ≤50m/s |  |  |
| Fan blade diameter/height | 0.8m/1.85m |  | 0.8m/2.15m |
| Number of blades | 10 |  | 12 |
| Blade material | Aluminum alloy |  |  |
| Fuselage material | Aluminum alloy |  |  |
| Magnetic steel material | NdFeB |  |  |
| Generator type | Three -phase AC permanent magnet levitation generator |  |  |
| Control mode | Electromagnetic brakes |  |  |
| Wind direction adjustment | 360° automatic windward wind |  |  |
| Service life/Degree of protection | ≥ 20 yearsIP54 |  |  |
| Net weight/gross weight | 65kg/75kg | 70kg/80kg | 80kg/90kg |
| Packing size | 1900*380*320mm |  | 2200*380*320mm |

Red Lantern Type	White Lantern Type- permanent magnet

| Model | R-50 | R-100 | R-200 | R-300 |
| --- | --- | --- | --- | --- |
| Rated power | 50W | 100W | 200W | 300W |
| Rated voltage | 12V/24V |  |  |  |
| Start the wind speed | 1.5m/s |  |  |  |
| Rated wind speed | 12m/s |  |  |  |
| Safe wind speed | ≤50m/s |  |  |  |
| Fan blade diameter/height | 0.9m/0.6m |  |  |  |
| Number of blades | 5/4 |  |  |  |
| Blade material | nylon |  |  |  |
| Fuselage material | Aluminum alloy |  |  |  |
| Magnetic steel material | NdFeB |  |  |  |
| Generator type | Three -phase AC permanent magnet generator |  |  |  |
| Control mode | Electromagnetic brakes |  |  |  |
| Wind direction adjustment | 360° automatic windward wind |  |  |  |
| Service life/Degree of protection | ≥ 20 yearsIP54 |  |  |  |
| Net weight/gross weight | 9kg/10kg | 9.5kg/10.5kg | 10kg/11kg | 11kg/12kg |
| Packing size | 61*44*30cm |  |  |  |

Type HL	Type BHL-treasure gourd

| Model | HL-50 | HL-100 | HL-200 | HL-300 |
| --- | --- | --- | --- | --- |
| Rated power | 50W | 100W | 200W | 300W |
| Rated voltage | 12V/24V |  |  |  |
| Start the wind speed | 1.5m/s |  |  |  |
| Rated wind speed | 12m/s |  |  |  |
| Safe wind speed | ≤50m/s |  |  |  |
| Fan blade diameter/height | HL=0.8m/0.65mBHL=0.8m/0.85m |  |  |  |
| Number of blades | 5 |  |  |  |
| Blade material | nylon |  |  |  |
| Fuselage material | Aluminum alloy |  |  |  |
| Magnetic steel material | NdFeB |  |  |  |
| Generator type | Three -phase AC permanent magnet generator |  |  |  |
| Control mode | Electromagnetic brakes |  |  |  |
| Wind direction adjustment | 360° automatic windward wind |  |  |  |
| Service life/Degree of protection | ≥ 20 yearsIP54 |  |  |  |
| Net weight/gross weight | 9kg/10kg | 9.5kg/10.5kg | 10kg/11kg | 11kg/12kg |
| Packing size | BHL=60*38*38cmHL=60*38*33cm |  |  |  |